Classes

Line segment class

- Profile variable: Consists of a list of ID, OD, wall thickness, distance-from-start, and elevation. Can alternatively be defined with hydraulic diameter D_h and flow_area in lieu of an ID or OD+wall thickness. Properties apply from each point forward in the segment.

- Roughness variable (eps) : absolute roughness of the entire segment

- Optional wall thermal conductivity variable (k) – Thermal conductivity pipe wall of the entire segment – nothing is done with this variable now, but later support will be added for heat transfer using this variable.

- Has dP function to calculate differential pressure from friction and elevation change across this segment given an Incompressible_Fluid object and a flow rate. Steps through segment and calculates the friction and expansion/contraction pressure drop at each segment, as well as the pressure drop due to acceleration/deceleration with the Bernoulli equation. Negative means pressure at end of slice is lower than pressure at beginning.

For each slice:

dP_slice = dP due to friction + dP due to elevation change + (dP due to expansion/contraction and velocity change from previous segment if not the first segment).

Note: If noncircular (i.e. if defined with D_h and flow_area instead of Di) then ignore dP due to expansion/contraction)

dP = sum of dP_slice over all slices

Bend class

- Internal diameter variable (Di)

- Bend angle variable (ang_deg): bend angle, in degrees

- Bend diameters variable (bend_dias) : Bend diameter in multiples if internal diameter (i.e. standard long-radius 1.5 * Di bend : bend_dias = 1.5)

- dP function that calls dP_bend

Contraction/expansion class

- Upstream diameter variable (Di_US)

- Downstream diameter variable (Di_DS)

- dP function that calls dP_contraction_expansion

Incompressible_Fluid class

- Density variable (rho)

- Viscosity variable (mu)

Global functions

dP_friction

- Calculates permanent friction pressure drop for a given length slice

dP_bend

- Calculates permanent friction pressure drop for a pipe bend

dP_contraction_expansion

- Calculates permanent friction pressure drop for an expansion or contraction